BIOS AND POST TROUBLESHOOTING THEORY

1. Five Common BIOS / POST Error Messages

| No. | BIOS / POST Error Message | Hardware / Configuration Issue |
| --- | --- | --- |
| 1 | CMOS Checksum Error – Defaults Loaded | Weak/dead CMOS battery or corrupted BIOS/CMOS settings. |
| 2 | Keyboard Error or No Keyboard Present | Keyboard is disconnected, faulty, or the keyboard port/controller has a problem. |
| 3 | Memory Size Error | RAM may be incorrectly installed, faulty, or the BIOS configuration does not match the installed memory. |
| 4 | No Boot Device Found | HDD/SSD may not be detected, the boot drive may have a problem, or the BIOS boot order may be incorrect. |
| 5 | BIOS ROM Checksum Error | BIOS firmware may be corrupted or the BIOS ROM may have a problem. |

2. AMI BIOS — Three Beeps

In a traditional AMI BIOS beep-code scheme, 3 beeps indicate a base-memory failure, specifically a failure involving the first 64 KB of system memory. This generally points toward a RAM/memory problem or, less commonly, the motherboard's memory circuitry. (Pearson Technology Group)

Two troubleshooting steps:

Reseat the RAM:
Turn off and unplug the PC, remove the RAM module(s), clean/check the contacts if appropriate, and firmly reinstall them. A motherboard manual may specify the correct slot.

Test the RAM individually:
If there are multiple RAM modules, test one module at a time and try the recommended memory slot. If one module consistently causes the problem, it may be faulty. A motherboard manual should be consulted because beep codes can vary by BIOS/motherboard model.                                                          
Conclusion:
3 AMI beeps + no display → first suspect RAM/memory.

3. BIOS Setup Screenshot and Identification

For this question, you need to access the BIOS/UEFI on an available PC and take your own screenshot/photo, because the motherboard manufacturer and BIOS version depend on the particular computer.

Steps:

Restart the PC.

During startup, repeatedly press the BIOS setup key, commonly F2, Delete, F10, or Esc depending on the manufacturer.

Find the Main, Information, or System Information page.

Locate:

Motherboard/System Manufacturer

BIOS/UEFI Version

Take a clear screenshot/photo of that screen.

Assignment format:

Motherboard Manufacturer: [Write the manufacturer shown on your PC]
BIOS Version: [Write the BIOS version shown on your PC]
BIOS Screen: Insert your screenshot here.

Important: Don't copy a random BIOS screenshot from the internet—the question asks you to identify the information from an actual available PC or virtual machine.

4. “CMOS Checksum Error – Defaults Loaded”

Meaning:
This message means that the BIOS detected that the stored CMOS configuration data does not match the expected checksum. As a safety measure, the BIOS loads its default settings instead of using the saved configuration. A weak/dead CMOS battery is a common cause, although corrupted BIOS settings or an unsuccessful BIOS update can also cause the problem. (HP Support)

How to resolve it:

Turn off the PC and disconnect the power before opening the case.

Start the PC and enter the BIOS/UEFI Setup.

Select Load Defaults / Optimized Defaults / Setup Defaults.

Check important settings such as date, time, and boot order.

Save the settings and restart the PC.

If the error returns repeatedly, check/replace the motherboard's CMOS/RTC battery, commonly a CR2032 in desktop PCs.

After replacing the battery, enter BIOS again and set the correct date, time, and required boot settings, then save and exit. (HP Support)

Easy way to remember:

CMOS Checksum Error → BIOS settings are not valid → Load Defaults → If it returns, check/replace CMOS battery.