COMPONENT -SPECIFIC TOUBLESHOOTING PRACTICAL

1. RAM Reseating — Step-by-Step Documentation

Objective: To remove and re-insert a RAM module and check whether it is properly recognized.

Steps Performed

Shut down the computer completely and disconnect the power cable/charger.

Press the power button for a few seconds to discharge remaining power.

Open the desktop cabinet or laptop's RAM access panel, if the RAM is accessible.

Locate the RAM slots on the motherboard.

Carefully release the locking clips on both sides of the RAM module.

Hold the RAM module by its edges and remove it without touching the gold contacts.

Inspect the RAM and slot for dust, dirt, or physical damage.

Align the notch on the RAM with the corresponding notch in the slot.

Press the RAM module firmly and evenly until the locking clips click into position.

Close the cabinet/access panel and reconnect the power.

Start the computer and check whether it boots normally and recognizes the installed RAM.

Issues Observed

Observation: The RAM module was properly seated after reseating. No physical damage was observed. The computer started normally and the memory was detected by the system.

Safety: Never force the RAM module. The notch must be aligned correctly before inserting it.

2. GPU Troubleshooting Checklist

| Step | What to Check | What to Do |
| --- | --- | --- |
| 1. Identify the symptom | No display, screen artifacts, crashes, or black screen | Note exactly when the problem occurs. |
| 2. Check monitor and cable | HDMI/DisplayPort cable and monitor | Try another cable, port, or monitor if available. |
| 3. Check GPU connections | GPU seating and power connectors | Turn off and unplug the PC, then check that the GPU is firmly seated and PCIe power cables are connected. |
| 4. Check display output | Monitor connected to the correct port | For a dedicated GPU, connect the display cable to the GPU's output rather than the motherboard output when appropriate. |
| 5. Check drivers | Graphics driver problems | Check Device Manager/appropriate system tools and reinstall or update the correct GPU driver if needed. |
| 6. Check temperature | GPU overheating | Check whether the GPU fan/cooling system is operating correctly and clean accumulated dust if appropriate. |
| 7. Test the GPU | Hardware stability | Run a suitable graphics test/game and watch for crashes, artifacts, or loss of display. |
| 8. Confirm the fix | Original problem has disappeared | Restart the PC and test the system again under normal and heavier graphics use. |

Final Confirmation

The GPU can be considered working normally when the PC displays correctly, the GPU is detected, there are no abnormal graphical artifacts/crashes, and the system remains stable during testing.

3. Storage Issue Simulation — SATA/NVMe Drive

Objective: To simulate a storage connection problem by disconnecting and reconnecting a storage drive.

Before Disconnecting

Shut down the test computer completely.

Disconnect the power.

Locate the SATA SSD/HDD or NVMe SSD.

Carefully disconnect the SATA data/power connection, or remove the NVMe drive according to the hardware manufacturer's procedure.

Error/BIOS Observation

If the disconnected drive contains the operating system, the PC may show messages such as:

“No boot device found”
“Boot device not found”
“Operating System not found”

In BIOS/UEFI, the disconnected drive may not appear in the Storage/SATA/NVMe information or Boot list.

After Reconnecting

Turn the computer off again.

Reconnect the SATA data/power cables, or correctly reinstall the NVMe drive.

Start the computer.

Enter BIOS/UEFI if necessary.

Check whether the drive appears in the storage/boot-device list.

Continue booting into the operating system.

Example Observation

| Condition | Observation |
| --- | --- |
| Drive disconnected | SSD/HDD not detected in BIOS; boot error displayed. |
| Drive reconnected | Drive appears in BIOS and PC boots normally. |

Conclusion: The test demonstrates that a loose/disconnected storage connection can prevent the BIOS from detecting the drive and can cause a boot failure.

4. PC Randomly Restarts — Which Component to Check First?

Among RAM, GPU, and storage, I would check the RAM first.

Reason

RAM problems can cause system crashes, freezes, unexpected restarts, boot problems, or memory-related errors. RAM is also relatively easy to reseat and test compared with some other hardware.

RAM Diagnostic Steps

1. Shut down the PC safely
Turn off the PC, disconnect the power cable, and allow the system to cool.

2. Reseat the RAM
Open the cabinet, release the RAM clips, remove the module, inspect it, and reinstall it firmly.

3. Check for multiple RAM modules
If two or more modules are installed, test them one at a time to determine whether one module causes the restart.

4. Check the RAM slots
Try the known-good RAM module in the motherboard's recommended slot. Check the motherboard manual for the correct configuration.

5. Observe the system
Start the PC and perform the same type of activity that previously caused the restart.

6. Confirm the fix
If the random restarts stop after reseating or replacing a problematic RAM module, the memory connection/module was a likely cause.

Simple Diagnostic Order

Random Restart → Check RAM → Check GPU → Check Storage

If the RAM checks out, then move to the GPU's power/connection/temperature and finally the storage drive and its connections.