DIAGNOSING HARDWARE FAILURES THEORY

1. Five Common Desktop PC Hardware Issues

| No. | Hardware Issue | Typical Symptom |
| --- | --- | --- |
| 1 | RAM failure | PC may not boot, show no display, or give repeated beep sounds. |
| 2 | Power Supply (SMPS) failure | PC does not turn on, or suddenly switches off/restarts. |
| 3 | Hard Disk/SSD failure | PC becomes very slow, cannot boot, or shows “No boot device found.” |
| 4 | CPU overheating | PC shuts down or restarts automatically, especially during heavy use. |
| 5 | Graphics card/display problem | Monitor shows a blank screen, distorted graphics, or no display. |

2. Laptop Not Charging — Step-by-Step Troubleshooting

Scenario: The laptop does not charge even when the charger is connected.

Check the wall socket – Plug another device into the socket to confirm that power is available.

Inspect the charger and cable – Look for cuts, burns, loose connections, or damaged insulation.

Check the charging connector – Make sure the charger plug fits firmly into the laptop's charging port.

Check the charger output – Use a multimeter set to DC voltage and measure the charger's output. Compare the reading with the voltage printed on the charger label.

Check the charging port – Look for dirt, bent pins, or a loose port. Do not force the connector.

Check the battery connection – If the laptop has an accessible battery, make sure its connector is properly seated.

Test with a known-good compatible charger – If available, try another charger with the correct voltage and connector.

Check for overheating or burning smell – If there is unusual heat, smell, or visible damage, stop testing and have the charger/laptop inspected.

Conclusion: If the charger has the correct output but the laptop still does not charge, the problem may be with the charging port, battery, charging circuit, or motherboard.

Safety: Do not open or probe inside the charger itself. Only measure the low-voltage output at the charger's connector.

3. Testing a USB Phone Charger with a Multimeter

Expected USB voltage: Approximately 5V DC, with 4.75V–5.25V being the specified range for a standard 5V USB supply.

Steps:

Set the multimeter to DC Voltage (V⎓).

Set the range to a value above 5V, such as 20V DC, if using a manual-range meter.

Connect the charger to a power socket.

Carefully place the black probe on the USB ground (GND).

Place the red probe on the USB +5V/VBUS contact.

Read the voltage shown on the multimeter.

Example record for the assignment:

Measured voltage: 5.08V DC
Expected range: 4.75V–5.25V
Result: Within the expected range — charger output is normal.

If you are using a real charger, replace 5.08V with your actual multimeter reading.

4. Testing the ATX Power Supply 12V Rail

Safety: Disconnect the ATX power connector from the motherboard before testing. Do not open the power-supply casing.

Steps:

Shut down the PC and disconnect the AC power cable.

Disconnect the main ATX power connector from the motherboard.

Set the multimeter to DC voltage (V⎓).

Select a range above 12V, such as 20V DC, if required.

Identify the yellow wire (+12V) and black wire (GND) on the ATX connector.

Touch the black probe to the black wire.

Touch the red probe to the yellow wire.

Read and record the voltage displayed.

Example measurement:

Measured 12V rail: 12.08V DC
Expected value: Approximately 12V
Result: Normal.

If the voltage is below 11.5V

A reading below 11.5V can indicate a weak or faulty power supply (SMPS/PSU) or an unstable 12V rail. This could contribute to random shutdowns, restarting, or system instability.

Easy memory tip:
Yellow = +12V | Black = Ground | Multimeter = DC Voltage.